INSTITUTE OF ELECTRICAL AND ELECTRONICS ENGINEER

CHENNAI-600025

STUDENT PROJECT SCHEME

Proposals for 2023-2024

Submitted under

STUDENT PROJECT PROPOSAL

Title of the Project

Solar Powered Robot For Solid Waste Collection On Water Surfaces

Batch Members

ABDUL RAHMAN N    (99345492)

SARAVANAN B            (99340054)

ESAKKIMUTHU M        (99341723)

NIKISH R                      (99339990)

Under The Guidance of

Dr. C. SOWTHILY / Assistant Professor

DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING

ROHINI COLLEGE OF ENGINEERING AND TECHNOLOGY

(Approved by AICTE & Affiliated to Anna University, Chennai)

Near Anjugramam Junction, Kanyakumari Main Road, palkulam, Variyoor-629401

Tamilnadu, India.

Phone:04652 266665, 266288 Email: admin@rcet.org.in |website: www.rcet.org.in

ABSTRACT:

In response to increasing water pollution from solid waste accumulation, we propose a novel solution using robotics and renewable energy. Our project focuses on designing a solar-powered robot capable of autonomously collecting waste on water surfaces. Integrating advanced technologies such as Arduino microcontrollers, Raspberry Pi units, sensors (ultrasonic, IR, moisture), a Raspberry Pi camera, and solar panels, we aim to optimize obstacle detection, waste identification, and autonomous operation. Field tests will evaluate the robot's effectiveness in mitigating pollution and scalability for widespread deployment, contributing to environmental conservation efforts.

Keywords: Ultrasonic sensor, IR sensor, Moisture sensor, Raspberry Pi, Arduino.

INTRODUCTION:

The world's water bodies face an escalating crisis of pollution, with marine debris posing a significant threat to aquatic ecosystems, wildlife, and human health. From plastic bottles to industrial waste, the accumulation of solid waste on water surfaces not only degrades the aesthetic appeal of natural environments but also disrupts delicate ecosystems, leading to long-term environmental consequences. In response to this pressing challenge, our project aims to develop an innovative solution leveraging robotics and renewable energy to tackle the issue of solid waste accumulation on water surfaces.

The concept of utilizing robotics for environmental cleanup is not new, but the integration of solar power presents a novel approach to enhancing the sustainability and autonomy of such systems. By harnessing the abundant energy of the sun, our solar-powered robot seeks to minimize its environmental footprint while maximizing its operational efficiency. Through the strategic integration of advanced technologies such as Arduino microcontrollers, Raspberry Pi computing units, and a suite of sensors and actuators, our project endeavors to create a versatile and adaptable platform capable of autonomously navigating water bodies and collecting solid waste.

At the heart of our project lies the vision of a cleaner and healthier environment, where water surfaces are free from the scourge of pollution, and aquatic ecosystems thrive in their natural state. By combining the power of robotics, renewable energy, and environmental stewardship, we aspire to make a tangible difference in the ongoing battle against marine pollution. Through collaboration with stakeholders from government agencies, environmental organizations, and local communities, we seek to deploy our solar-powered robot in real-world scenarios, demonstrating its effectiveness in mitigating the impacts of solid waste pollution on water ecosystems.

OBJECTIVE :

Our project aims to develop a solar-powered robot capable of efficiently collecting solid waste on water surfaces. The primary objective is to design and construct a prototype that integrates advanced technologies, including Arduino microcontrollers, Raspberry Pi computing units, and a suite of sensors and actuators. We will optimize the robot's energy efficiency by incorporating solar panels for renewable energy and implementing power-saving algorithms. Additionally, we aim to enable autonomous operation, allowing the robot to navigate water bodies, detect obstacles, identify waste, and execute collection maneuvers without human intervention. Field testing will validate the robot's performance and effectiveness in real-world scenarios, contributing to the advancement of sustainable technologies for environmental conservation.

LITERATURE REVIEW:

| TITLE OF THE PAPER | JOURNAL NAME/YEAR OF PUBLICATION | FINDINGS | LIMITATIONS |
| --- | --- | --- | --- |
| Development of a Solar-Powered Autonomous Robot for River Cleanup | Environmental Technology, 2020 | The solar-powered autonomous robot effectively navigated river environments and collected debris, demonstrating its potential for river cleanup operations. | Challenges were encountered in maneuvering the robot in fast-flowing rivers, leading to limitations in debris collection efficiency. |
| Solar-Powered Unmanned Surface Vehicle for Coastal Waste Management | Ocean Engineering, 2019 | The solar-powered unmanned surface vehicle autonomously collected waste along coastal areas, significantly reducing cleanup time and costs. | Limited onboard storage capacity constrained the robot's ability to collect large volumes of debris before requiring manual intervention. |
| Autonomous Marine Debris Collection Using Solar-Powered Drones | Journal of Field Robotics, 2021 | Solar-powered drones efficiently identified and collected marine debris from remote coastal regions, demonstrating versatility and scalability. | Adverse weather conditions, such as high winds and heavy rainfall, hampered drone operations and limited collection efficiency. |
| Robotic Waste Collector for Urban Water Bodies: A Case Study | Waste Management & Research, 2020 | The robotic waste collector effectively removed debris from urban water bodies, improving water quality and enhancing aesthetic appeal. | Limited battery life restricted the operational range of the robot, necessitating frequent recharging and limiting continuous operation. |
| Solar-Powered Robot for Solid Waste Collection in Riverine Ecosystems | Ecological Engineering, 2018 | The solar-powered robot successfully navigated riverine ecosystems, collecting solid waste and preventing further pollution downstream. | Challenges were encountered in maintaining consistent communication between the robot and control center, leading to occasional disruptions in operation. |

METHODOLOGY :

Phase 1: Conceptualization and Design

- Define project objectives, scope, and requirements.

- Conduct background research on existing technologies and environmental challenges.

- Outline the architecture and key features of the solar-powered robot.

Phase 2: Component Selection and Procurement

- Identify necessary components based on design specifications.

- Evaluate and select components for compatibility, performance, and reliability.

- Procure components from reputable suppliers ensuring timely delivery.

Phase 3: Prototyping and Assembly

- Fabricate mechanical components using 3D printing or machining techniques.

- Integrate electronics and sensors into the robot's framework.

- Develop software for microcontrollers and computing units to enable functionality.

Phase 4: Testing and Optimization

- Conduct rigorous testing to assess navigation, obstacle detection, and waste collection capabilities.

- Measure and record performance metrics such as energy efficiency and reliability.

- Optimize hardware, software, and control algorithms based on test results and feedback.

Phase 5: Field Deployment and Evaluation

- Deploy the solar-powered robot in real-world water environments for field testing.

- Collaborate with stakeholders to assess performance and effectiveness in waste collection.

- Iterate on design and functionality based on field test results for continuous improvement.

COMPONETS SPECIFICATIONS:

Solar Panel:

The solar panel serves as the primary power source for the robot, converting sunlight into electrical energy. It typically consists of multiple solar cells interconnected to generate the required voltage and current. The power output of the solar panel determines the amount of energy it can provide to charge the robot's battery and power its operations.

Raspberry Pi:

The Raspberry Pi is a credit-card-sized single-board computer that provides computing capabilities to the robot. It serves as the brain of the system, handling tasks such as data processing, decision-making, and communication. With its powerful processor and various connectivity options, the Raspberry Pi enables the robot to perform complex operations and interact with its environment.

Arduino Uno:

The Arduino Uno is a microcontroller board based on the ATmega328P chip, providing the robot with real-time control over its sensors and actuators. It manages low-level tasks such as sensor data acquisition, motor control, and interfacing with external devices. With its versatile I/O pins and easy-to-use programming environment, the Arduino Uno enables rapid prototyping and customization of the robot's behavior.

Ultrasonic Sensor:

The ultrasonic sensor emits high-frequency sound waves and measures the time taken for the sound waves to reflect off an object and return to the sensor. By calculating the distance to the object based on the time delay, the ultrasonic sensor enables the robot to detect obstacles and navigate its environment safely.

IR Sensor:

The IR sensor detects infrared radiation emitted or reflected by objects in its field of view. It can be used for various purposes, including obstacle detection, object tracking, and line following. By detecting changes in infrared radiation, the IR sensor helps the robot perceive its surroundings and react accordingly.

Moisture Sensor:

The moisture sensor measures the moisture content of the soil or water, providing feedback on environmental conditions. It typically employs a resistance-based or capacitive sensing method to detect changes in moisture levels. By monitoring moisture levels, the sensor enables the robot to assess the water quality and identify areas with high concentrations of pollutants or waste.

Motor Driver L298N:

The L298N motor driver is a dual H-bridge motor driver that controls the speed and direction of DC motors. It provides the necessary power amplification and protection circuitry to drive the robot's motors efficiently and safely. With its PWM control and bidirectional motor control capabilities, the L298N motor driver enables precise control over the robot's movement.

DC Motor:

The DC motor converts electrical energy into mechanical motion, driving the propulsion the robot. It typically consists of a rotor and a stator, with the rotor rotating when current is applied to the stator windings. By varying the voltage and polarity applied to the motor, the robot can move forward, backward, turn, and perform as required.

Bluetooth Sensor:

The Bluetooth sensor enables wireless communication between the robot and external devices such as smartphones or tablets. It uses Bluetooth technology to establish a communication link, allowing users to control the robot remotely, send commands, and receive data. With its low-power consumption and wide compatibility, the Bluetooth sensor facilitates flexible and convenient interaction with the robot.

TP4056:

The TP4056 is a lithium battery charger module used to charge the robot's battery using the solar panel. It provides linear charging with overcharge and over-discharge protection, ensuring the safe and efficient charging of lithium-ion batteries. With its compact size and simple interface, the TP4056 module offers a reliable solution for managing the robot's power supply.

Raspberry Pi Camera:

The Raspberry Pi Camera Module is a high-resolution camera that captures images and videos of the robot's surroundings. It connects to the Raspberry Pi via the Camera Serial Interface (CSI) and provides visual data for tasks such as object detection, navigation, and environmental monitoring. With its compact size and plug-and-play compatibility, the Raspberry Pi Camera Module offers a versatile imaging solution for the robot.

Battery

HC-05 Bluetooth Sensor

Ultrasonic Sensor

IR Sensor

Moisture Sensor

Raspberry Pi Camera

Motor Driver

Mobile Application

Arduino

Raspberry Pi

TP4056

Solar Panel

Source

Motor 1

Motor 4

Motor 3

Motor 2

CERTIFICATE

This is to certify that Mr. ABDUL RAHMAN N is a bonafide final year student of U.G. Engineering courses of our college and it is also certified that two copies of utilization certificate and final report along with seminar paper will be sent to the Council after completion of the project by the end of May 2024.

CERTIFICATE

This is to certify that Mr. SARAVANAN B is a bonafide third year student of U.G. Engineering courses of our college and it is also certified that two copies of utilization certificate and final report along with seminar paper will be sent to the Council after completion of the project by the end of May 2024.

CERTIFICATE

This is to certify that Mr. ESAKKIMUTHU M is a bonafide third year student of U.G. Engineering courses of our college and it is also certified that two copies of utilization certificate and final report along with seminar paper will be sent to the Council after completion of the project by the end of May 2024.

CERTIFICATE

This is to certify that Mr. NIKISH R is a bonafide third year student of U.G. Engineering courses of our college and it is also certified that two copies of utilization certificate and final report along with seminar paper will be sent to the Council after completion of the project by the end of May 2024.

| Signature of the Guide | Signature of the HOD | Signature of the Principal/ Head of the Institution |
| --- | --- | --- |